AI Scene CAMM:
Web-based platform to assist clinicians in measuring environmental features during real-world travel

# Producer notes

{Notice to the reader about accessibility: This document meets the Government of Québec SGQRI 008-2.0 standard to be accessible to anybody, disabled or not. All notices between braces are alternative texts for images, abbreviations or to describe any other information conveyed by sensory characteristics that transmits information, indicates an action, prompts a response, or distinguishes a visual element.

This document was made accessible by Service Adaptation de l'Information of Institut Nazareth & Louis-Braille being part of Centre Intégré de Santé et de Services Sociaux de la Montérégie-Centre.

955, d'Assigny – door 139
Longueuil (Québec) J4K 5C3
Phone: 450 463-1710, ext. 159346
Toll free: 1 800 361-7063, ext. 159346
Fax: 450 670-0220
E-mail: braille.inlb@ssss.gouv.qc.ca

Notes: Make sure you modified your Jaws' reading parameters by activating language detection and most punctuations reading.}

# Special symbols

{__>} rightwards arrow

{<__} leftwards arrow

{<} downwards arrow

{><} up down arrow

{<__>} left right arrow; z notation relation

{r} {/r} frames red colored elements

{g} {/g} frames elements of green color

# Navigation links

Producer notes

Special symbols

Navigation links

AI Scene CAMM: Web-based platform to assist clinicians in measuring environmental features during real-world travel

HDM-DCP (PPH) Model of disability:

Measuring environmental disability:

AI Scene CAMM concept:

Experimental Approach:

AI Scene CAMM: Objective scene measurements

AI Scene CAMM: Video note-taking

Safety and confidentiality

Future Work:

Demo: AI Scene CAMM

Acknowledgments:

{Slide 1}

# AI Scene CAMM:
Web-based platform to assist clinicians in measuring environmental features during real-world travel

Sergio Mejia-Romero
Nemargut's Lab

{Logo École d'optométrie. Université de Montréal}

{Logo NEMARGUT LAB. Université de Montréal}

{* * *}

February 3rd, 2026

23e Symposium scientifique sur l'incapacité visuelle et la réadaptation

{Slide 2}

## HDM-DCP (PPH) Model of disability:

The ability of an individual to perform a task is based on their person capacity and environmental characteristics.

Measuring personal capacity:

  - Vision: ETDRS Chart, MARS Contrast sensitivity, etc.

  - Hearing: Audiogram, etc.

  - Environment: ?

{Illustration}

This image is made up of two squares that illustrate personal factors and environmental factors as well as a rectangle at the bottom of the image that illustrates the life habits found in the DCP model. The relationships between its forms are indicated by arrows that go in both directions. In addition, there is another square above personal factors that represents the risk factors. This square is accompanied by a unidirectional arrow pointing to personal factors.

Risk Factors [cause]
{<}

Personal Factors
{><}

Organic Systems [Integrity {<__>} Deficience]

Capabilites [Ability {<__>} Disability]

{><}

Interaction
{><}

Life Habit [Social Participation {<__>} Handicap Situation]

{* * *}

Risk Factors [cause]
{<}

Environmental Factors [Facilitator {<__>} Obstacle]
{><}

Interaction
{><}

Life Habit [Social Participation {<__>} Handicap Situation]

{/Illustration}

{Slide 3}

## Measuring environmental disability:

Both the person capacity and the environment are inherently linked

How to measure this when it is always changing?

Sound

Lighting

Moving objects

{Illustration}

Person capacity

{gradient color bar: green {__>} yellow {__>} red}

{g}Very capable/Enabling{/g} {<__>} {r}Unable/Very disabling{/r}

| Environmental characteristics | Environmental Disability Scale | Environmental Impact Score |
| --- | --- | --- |
| {Illustration not described} | {yellow {<__} green} | Negative (facilitates task performance) |
| {Illustration not described} | {green {__>} orange} | Low (reduces task performance) |
| {Illustration not described} | {yellow {__>} red/orange} | High (hinders task performance) |

{/Illustration}

{Slide 4}

## AI Scene CAMM concept:

Quantifies the environmental scene difficulty using:

  - Luminance & contrast

  - Clutter & crowding

  - Motion

  - Object detection

  - Timing

Need to develop and validate the models

{Slide 5}

## Experimental Approach:

Recruiting participants:

  - VI, HI, DSI & controls

Inputs:

  - Short online videos + laboratory trials

Objective metrics:

  - Visual: luminance, contrast, entropy, edge density, optical flow, object detection

  - Auditory: Background, Volume, Frequency, Dynamics, traffic, saliency

{Slide 6}

## AI Scene CAMM: Objective scene measurements

{Video not described. Please refer to the source document to view it.}

{Slide 7}

## AI Scene CAMM: Video note-taking

{Video not described. Please refer to the source document to view it.}

{Slide 8}

## Safety and confidentiality

Server in Nemargut laboratory at the Université de Montréal

Restricted access (dual-factor identification)

Face blurring

No shared access

No sharing of data to third parties (i.e. Google, Meta, etc.)

{Slide 9}

## Future Work:

Integration of AI models (traffic, crowd, clutter...)

Open Platform for clinical testing

Integration of Clinical / Rehab tools to measure performance

Accessibility index

{Slide 10}

## Demo: AI Scene CAMM

{QR code}

{/QR code}

{Slide 11}

## Acknowledgments:

Collaborating Lab:

  - Prof. Yingzi Xiong
Lions Vision Research and Rehabilitation Center, Wilmer Eye Institute, Johns Hopkins University.

Contributors:

  - Qingying Gao.

  - Andre DeGrenier.

Special thanks:

  - Collection team:

    - Deborah Annan

    - Prajjol Raj

    - Safiata Kabore

Funding/Support:

  - SSHRC
Research to prevent blindness

{Illustration not described}